Installation Instructions

Dual Dipole Antenna

Partnership between

tinyurl.com/SunRISEGRLAntennaInstructions

Table of Contents

Installation Materials and Tools …………………………………..… Page 3 Before Getting Started ……………………………….……………….. Page 3 Antenna Installation

Step 1 ……………………………………………………………… Page 4 Step 2 ……………………………………………………………… Page 5 Step 3 ……………………………………………………………… Page 6 Step 4 ……………………………………………………………… Page 7 Step 5 ……………………………………………………………… Page 8 Step 6 ……………………………………………………………… Page 9 Step 7 ……………………………………………………………… Page 10

Step 8 ……………………………………………………………… Page 11 Software Setup and Data Collection Guide

Software Guide Steps 1-5 ………………………………………. Page 13 Software Guide Steps 6-10 ……………………………..………. Page 14 Software Guide Steps 11-15 ……………………………………. Page 15 Software Guide Steps 16-18 ……………………………………. Page 16

Antenna Disassembly and Storage ……………………..……..…... Page 17 Acknowledgements ……………………………………………….…... Page 18

Contributors

Tremba, D. 1(djtremba@umich.edu); Liu, C. 1; Akhavan-Tafti, M. 1; Soni, S. L. 1; Higgins, C. 2; Fung, S. 3 1 Department of Climate and Space Sciences and Engineering (CLaSP), University of Michigan, Ann Arbor, MI 2 Middle Tennessee State University, Murfreesboro, TN

3 NASA Goddard Space Flight Center, Greenbelt, MD

| Label | Quantity |
| --- | --- |
| A | 8 |
| B | 4 |
| C | 8 |
| D | 4 |
| E | 2 |
| F | 4 |
| G | 1 |

Installation Materials and Tools

Item

PVC pipes (1.52 m or 5 ft)

Mast End Caps (pre-assembled; a PVC cap with a bolt stuck through it) PVC Couplings

Mast Tops (pre-assembled; a short length of PVC pipe with eye bolts sticking  through it)

Coils of Nylon Rope (30.48 m or 100 ft)

Zip Ties (20.3 cm or 8 in)

Roll of Duct Tape (optional)

H 1 Roll of Electrical Tape or Coaxial Sealing Tape (optional) I 8 Ground Stakes

Dipoles (bare copper wire connected to a black insulator and coaxial cable socket in

J 2

the center; 7.10 m or 23 ft 3 in)

K 2 Coils of Coaxial Cable (12.53 m or 41 ft + 1.25 in) L 1 Coil of Coaxial Cable (25.06 m or 82 ft + 2.5 in) M 1 Phasing Cable (4.70 m or 15 ft + 5 in coil of coaxial cable) N 1 Female-Female Coaxial Adapter O 1 Power Combiner (small rectangular box with 3 coaxial cable sockets) P 1 SDR Receiver (black rectangular box) Q 1 SMA to BNC Adapter Cable R 1 BNC to Female Adapter S 1 Tape Measure

T 1 Compass

U 1 Utility Knife (preferred) or Scissors V 1 Permanent Marker

W 1 Rubber Mallet or Rock + Wood Block (optional) Before Getting Started

While setting up or tearing down, be aware of your surroundings to  avoid inflicting or receiving injuries that could result from swinging  around PVC pipes, tangled cords on the ground, etc. Exercise caution  when setting up on a roof or when dealing with electrical connections.

We recommend having at least 2, but preferably 3 or more people to  set up or take down the SunRISE GRL Antenna. Installation generally  takes 2-3 hours the first time around, but it is possible to complete  within half an hour or less with enough practice and/or more people.

If you are intending to frequently set up and take down the antenna,  the boxes used for shipping the antenna components can be great for  storing them when not in use. If you have no use for the boxes, please  recycle them and all other packaging, when practical.

For detailed instructions on antenna  installation, scan the QR code below:

tinyurl.com/SunRISEGRLFullInstructions

3

Nylon Rope (E)

Antenna Installation: Step 1

Tape Measure (S)

Scissors (U)

Scan QR code for video  demonstration

Duct Tape (G)

Permanent Marker (V)

a. Measure out 6 m

Tip: Get help from a second person to hold the

rope in place at one end.

b. Mark the rope at 6 m

OPTIONAL - Tightly wrap

marked rope in duct tape

c. Cut rope at the marked spot

d. Repeat Steps 1a-1c eight times

to make eight lengths of rope

tinyurl.com/SunRISEGRLInstructionsStep1

Lay out the rope flat on the  ground next to a tape measure.

If duct tape is available, wrap it  tightly around the spot you  marked on the rope two or three  times before cutting to prevent  fraying.

If you chose to wrap the rope in  duct tape, be sure that you keep  track of the spot you marked with  a sharpie and cut as close to that  location as possible.

4

Step 2

Mast Tops (D) PVC Pipes (A)

PVC Couplings (C)

Scan QR code for video  demonstration

tinyurl.com/SunRISEGRLInstructionsStep2

a. Insert PVC pipe into PVC  coupling

b. Insert a second PVC pipe into  the coupling

c. Attach a second coupling to  the top of the mast

Tip: Now that we have assembled them partially,  we will now refer to the connected PVC pipes as  “masts.”

d. Attach the mast top to the  second coupling

Tip: If you have issues getting the pieces to fit  together snugly, try forcing the top of the mast  against the ground and pushing downward on the  mast.

e. Repeat Steps 2a-2d four times  to make four masts

Be sure that there aren’t any  gaps between components of the  mast; push all the pieces

together until they can’t move  any further.

5

Guy Lines

Masts

Step 3

a. Make a bight in the rope,  leaving roughly 30 cm at the end

Tip: “Bight” is another word for a bend in a rope,  often used in knot tying.

b. Make a loop in the rope, then  thread the bight through that loop

c. Thread the bight through the  loop again and pull it tight

d. Attach loop to eyebolt

Note: The ropes must be attached to the lower  eyebolt–the upper eyebolt is reserved for the  dipole.

e. Repeat Steps 3a-3d such that  there are two guy lines attached to  each mast on the same eyebolt (as  shown)

Tip: Now that the ropes are attached to the masts,  from now on we’ll refer to them as “guy lines.”

Scan QR code for video

demonstration

tinyurl.com/SunRISEGRLInstructionsStep3

It’s necessary to learn how to tie  a double overhand loop in  order to attach the ropes to their  masts. If you have trouble  following the directions that  follow, feel free to view the video  linked in the QR code.

The ropes can be attached to the  eyebolts by passing the loop  through the eyebolt, then

passing the other end of the rope  through the loop in the rope.

6

Step 4

Masts + Guy Lines

Tape

Measure (S)

Permanent

Marker (V)

Scan QR code for video  demonstration

tinyurl.com/SunRISEGRLInstructionsStep4

a. Measure 440 cm of rope

b. Pull on the rope and mark it  at 440 cm

c. Make a bight in the rope at the  point you marked

d. Tie a double overhand loop (as  demonstrated on page 6)

e. Repeat steps 4a-4d for every  guy line

Have another person hold the  top of a mast tightly and line it up  such that the end of the bottom  eyebolt where the rope attaches  is at the location where the tape  measure reads zero.

Keep tension on the rope while  marking it; you can release the  tension as soon as you have  completed this.

The loop you tie should be at  least 2 cm long.These loops will  be used to attach the guy lines to  the ground stakes

7

Tape Measure (S)

Mast End Caps (B)

Step 5

Compass (T)

Scan QR code for video  demonstration

tinyurl.com/SunRISEGRLInstructionsStep5

a. Find a spot with at least 12x12  m (39.4x39.4 ft) of space oriented  N-S on two opposite sides

Note: Choose an area free of overhead  obstructions and as far as possible from tall  buildings, metal fences, and other metal  structures–especially power lines.

c. Mark where masts will go using  bottom pieces

Tip: If necessary, you can use a rock or rubber

An unobstructed view in the  direction of the Sun and/or

Jupiter is desirable–to have a  good view all year round, this  would be in the direction of the  equator relative to your latitude.

b. Use a compass and tape  measure to take measurements in  order to establish masts as shown  in the diagram to the left

Note: It is imperative that the antenna structure  maintains its orientation with respect to the cardinal  directions!

Note: You do not need to mark where the stakes  will go. The measurements given for the stake  positions are approximations, and it is not  necessary to place the stakes in these exact  positions.

As you measure out where each  mast is meant to go, insert the  bottom piece of the mast all the  way into the ground at the place

mallet to lightly pound the bottom piece into the  ground.

where the rest of it will be  attached later.

8

Electrical Tape (H) Zip Ties (F)

Dipoles (J)

Step 6

Masts + Guy Lines

12.53 m

Coaxial

Cable (K)

Scan QR code for video  demonstration

tinyurl.com/SunRISEGRLInstructionsStep6

a. Lay out masts

b. Make note of the location of the  “+” symbol

c. Attach dipole to eye-bolts  using zip ties

Note: Be sure not to make zip ties so tight that the  eyebolt touches the dipole–you should be able to fit  at least two or three fingers between the dipole and  the eyebolt.

d. Connect 12.53 m Coaxial Cable  to Dipole

Note: Connect the end of the cable with ferrite  toroids attached to it (these are small black  cylinders roughly 3 cm long).

OPTIONAL - Wrap coaxial  connections

Note: If you decide that it’s necessary to do this,  we recommend wrapping all coaxial connections

Lay out all four masts with the  end of each mast close to the  location where it’ll be inserted  later.

When you connect the dipoles to  their masts, make sure that this  symbol is on the west side of  both dipoles–this is absolutely  crucial.

Be sure that you are attaching  the dipoles to their masts such  that the dipole runs along the  E-W axis.

If available, it could be helpful to  cover the coaxial connections  using electrical tape or coaxial  sealing tape–especially if the  antenna will be exposed to

(anywhere that a cable screws onto a socket) with  the tape, not just the ones that are connected to the  dipole.

moisture during its 2-year  deployment.

9

Stakes(I)

Step 7

a. Insert mast into end cap

Tip: You will probably need to use your bodyweight  in order to get the mast to go all the way into the  cap.

b. Insert stakes into ground

Tip: While inserting the stakes, try to keep as much  tension on the guy lines as possible, as these will  provide most of the structural support for the  antenna.

c. Repeat the above steps for the  second mast.

Tip: The dipole should be fairly taut and parallel to  the ground, however some sagging is normal. If it is  not taut, try moving the stakes further from the  masts.

d. Repeat the above steps for the  remaining set of masts

Scan QR code for video

demonstration

tinyurl.com/SunRISEGRLInstructionsStep7

Have a second person hold one  of the masts vertical and insert  the foot of the mast into the end  cap embedded in the ground.

Loop the ends of the guy lines  around their stakes and insert  the stakes halfway into the  ground at a 45° angle with  respect to the mast, angled away  from the mast. The positions of  the stakes do not need to be  perfect.

Be sure that you are attaching  the dipoles to their masts such  that the dipole runs along the  E-W axis.

10

Power Combiner (O)

SDR

Receiver (F)

Adapter

Step 8

25.06 m

coaxial

Cable (L)

Scan QR code for video  demonstration

Cable (Q) Phasing Cable

(M)

BNC to F

Adapter (R)

F to F Adapter

(N)

a. Connect phasing cable using F

to F adapter only if antenna

latitude is beyond ± 30°

Tip: Although the ends of the adapter are different

lengths, either cable can be attached to either end.

b. Connect both cables to the

power combiner

Note: It doesn’t make a difference which socket

these two cables are connected to, however, you

must ensure that the cables are both connected on

the same side.

c. Connect 25.06 m cable to

socket of power combiner

DANGER: ALWAYS DISCONNECT COAXIAL

CABLE FROM THE POWER COMBINER OF THE

ANTENNA AFTER EACH USE IN CASE OF

LIGHTNING.

d. Connect BNC to F adapter

between 25.06 m cable and SMA to

BNC adapter cable

Tip: To make the BNC connection, line up the pins

with the track, force the adapter into the socket,

and twist the adapter.

e. Connect SMA to BNC adapter

cable to SDR receiver

Tip: The gold connector screws on the same way

as previous coaxial connections.

tinyurl.com/SunRISEGRLInstructionsStep8

Attach the 4.7 m phasing cable  to the 12.53 m coaxial cable  connected to the dipole that is  closer to the equator relative to  the other.

If you connected the phasing  cable, one of the cables you  connect here will be the other  end of the phasing cable, and  the other will be the remaining  12.53 m cable.

The female end of the adapter  screws onto the coaxial cable the  same way as any other

connection

11

Software Setup and Data Collection Guide

For detailed instructions on data

collection, scan the QR code below:

Note: Please refer to the full software setup guide for details; the full

manual is recommended to be displayed on electronic devices like a PC. tinyurl.com/SunRISEGRLFullDataInstructions

Data Transmission Materials and Tools

Label Quantity Item

A 1 SDRplay RSP1A receiver

B 1 USB type B to USB type A cable

C 1 Computer running Windows 7, 8, or 10 system

Required Software

SDRuno Radio-Sky Spectrograph

sdrplay.com/sdruno/ radiosky.com/specdownload.html

12

Steps 1-5

1. Go to the SDRplay website  download page and download and  install the software

2. Connect the RSP receiver to  your PC with the USB type B to  USB type A cable

3. Start SDRuno to verify that  your radio is working and tune the  radio signal

4. After tuning and verifying that  the SDRplay radio receiver is  working, close SDRuno

Note: You cannot use RSS (see below) and SDRuno simultaneously; you must close SDRuno in order to complete the steps that follow.

5. Download and install (in the default directory!) the latest version of Radio-Sky

Spectrograph (RSS)

Scan QR code for video

demonstration

tinyurl.com/SunRISEGRLDataStep1-5

To download SDRplay, follow  the QR code/link on page 12.  Scroll to the bottom of the page  and click on the download  button.

Set the gain to “maximum” and and set the mode to

“AM.”

Once again, follow the QR  code/link on page 12 to

download RSS. If the download  link fails, see the full manual

(linked on page 12) for an  alternative method.

13

Steps 6-10

Scan QR code for video  demonstration

tinyurl.com/SunRISEGRLDataStep6-10

6. Run Radio-Sky Spectrograph

as an administrator

Note: Ensure that the SDR receiver is properly

connected to your PC. The data collection will start

soon.

7. When you run RSS for the first time, click the Identity/Antenna menu and provide your local info

Note: Choose an observatory name that is  identifiable with your high school.

8. In RSS, Click on “Receiver” and select the SDRplay option

9. After selecting the SDRplay option, open

RSP1A_20211028.config from the pop-up window when asked to select a new configuration

10. Set the program priority

At a minimum, fill in your name,  observatory name, latitude and  longitude, and location.

Set the program priority to “High” and turn off the

auto-screen lock on your PC. These two steps help to stop

the data transmission from being unintentionally interrupted.

14

Steps 11-15

11. Start the data collection

12. When an SDRplay receiver is  selected in RSS and the Start  button is pushed, RSS will check  to see if SDRPlay2RSS is already  running

13. When the designated  observation time is finished, stop  the data transmission by clicking  the “Stop Saving” button

14. Save the data; we need the  data in both image (JPEG) and  binary code (SPS) session log file  formats

15. Load the SPS session log file that you saved

Scan QR code for video

demonstration

tinyurl.com/SunRISEGRLDataStep11-15

For troubleshooting and FAQs,  please refer to the full guide (linked  on page 12). Go to Step 11 ->  Some Key Tips for Normal  Operation.

The session log file is automatically named in the following format: [year-month-date-hour-minute second-observatory name].

The default file location is

Documents->SpecData->Observat ory name->month->date.

15

Steps 16-18

16. Click the double-arrow button to display the spectrogram of the entire observation session

17. Save the data as an image by  selecting the option shown

18. Make sure you collect the  data in all required formats and  inspect the data collected

Tip: Inspect the data in both the SPS and JPEG  formats.

Scan QR code for video

demonstration

tinyurl.com/SunRISEGRLDataStep16-18

16

Large box

1-2 small boxes and/or bags

Disassembly and Storage

Six ~30 cm lengths

of rope or other

Scan QR code for video  demonstration

flexible material

https://tinyurl.com/SunRISEGRLInstructionsTeardown

a. Remove stakes and masts from  ground

Tip: Have a second person hold the mast vertical  while you remove the stakes from the ground.

b. Disconnect Coaxial Cables

Note: You do not need to disconnect the female to  female adapter from the phasing cable, or the BNC  to female adapter from the 25.06 m cable.

c. Disassemble masts

Tip: Rather than pulling the mast apart, we  recommend twisting the mast apart and having  another person help you do so.

d. Storage

Tip: Store stakes and end caps separately from the  other parts of the antenna to prevent dirt buildup (a  small box or bag works great for this).

After removing the stakes from  the ground, pull the mast out of  the ground and lay it down on its  side. Do this for every mast.

Once done disconnecting all  coaxial cable connections, gently  coil up each cable individually  and wrap a rope or other

material around it to prevent it  from tangling.

Disconnect the 1.52 m PVC  pipes from each other; remove  the mast top. Coil the guy lines  around the mast top between the  two eyebolts and tuck in the  excess rope. Do not cut the zip  ties. Coil up the dipoles.

Any sizable box will work for  storing antenna components. Be  sure to store them in a cool, dry  place.

17

Acknowledgements: SunRISE GRL was sponsored by NASA grant  #AWD006989, and hosted at the Department of Climate and Space Sciences  and Engineering (CLaSP), University of Michigan College of Engineering, Ann  Arbor, MI. The SunRISE GRL team would like to thank 1) the RadioJOVE  team for providing the initial antenna design and providing feedback  throughout the development of this document (visit  https://radiojove.gsfc.nasa.gov/); and 2) the students and staff at Skyline High  School, Ann Arbor, MI for their support in testing the manual and providing  feedback. For more information, visit the SunRISE GRL webpage:  https://sunrise.umich.edu/